A project report on

ENHANCED AUTONOMOUS ROBOT WITH CENTRAL NAVIGATION SYSTEM

Submitted in the partial fulfillment of the requirements for the award of the degree of

BACHELOR OF TECHNOLOGY

IN

ELECTRICAL AND ELECTRONICS ENGINEERING

SUBMITTED BY

| B. Manikanta | 19MH5A0208 |
| --- | --- |

Under the esteemed guidance of

K. Manoz Kumar Reddy Mtech,(Ph.D)

Associate Professor

Department of Electrical and Electronics Engineering

ADITYA COLLEGE OF ENGINEERING

Approved by AICTE, Permanently Affiliated to JNTUK, & Accredited by NAAC Recognized by UGC under section 2(f) and 12(B) of UGC Act 1956, Approved by A.I.C.T.E, New Delhi

Aditya Nagar, ADB Road, Surampalem-533437

2021-2022

CERTIFICATE

DEPARTMENT OF ELECTRICAL AND ELECTRONICS ENGINEERING

ADITYA COLLEGE OF ENGINEERING

Approved by AICTE, Permanently Affiliated to JNTUK, & Accredited by NAAC Recognized by UGC under section 2(f) and 12(B) of UGC Act 1956, Approved by A.I.C.T.E, New Delhi

Aditya Nagar, ADB Road, Surampalem-533437

This is to certify that the project report entitled “ENHANCED AUTONOMOUS ROBOT WITH CENTRAL NAVIGATION SYSTEM” being submitted by.

| B. Manikanta | 19MH5A0208 |
| --- | --- |

In the partial fulfillment of the requirement for the award of the degree of the Bachelor of Technology in Electrical and Electronics Engineering from Aditya college of engineering. It is the record of bonafide work, carried out by them at Aditya College of Engineering.

Head of the Department

Sri. K.Manoz Kumarreddy M.Tech,(Ph.D)

Associate Professor

ACKNOWLEDGEMENT

The satisfaction that accompanies the successful completion of any task would be incomplete without the mention of the people who made it possible and whose constant guidance and encouragement crown all the efforts with success. The acknowledgment transcends the reality of formality.

We are thankful to our beloved guide cum Head of the Department Sri. K. M. K. REDDY, M.Tech(Ph.D),, M.Tech, Assistant Professor who has spared his valuable time and append novel ideas to guide me in limelight. I am indebted to him without whom I may not have culminated in the pinnacle of the project.

We are thankful to Dr.A.RAMESH, M.Tech, Ph.D., Principal, Aditya College of Engineering for providing the appropriate environmental required for this project.

We also thankful to all staff members, lab technicians, librarians and friends who are directly and indirectly helped me in the completion of the project with flying colors.

### With sincere regards

| B. Manikanta | 19MH5A0208 |
| --- | --- |

DECLARATION

We hereby declare that this project titled “ENHANCED AUTONOMOUS ROBOT WITH CENTRAL NAVIGATION SYSTEM” has been undertaken and this work is submitted to ADITYA COLLEGE OF ENGINEERING, Surampalem, affiliated to JAWAHARLAL NEHRU TECHNOLOGICAL UNIVERSITY, Kakinada in the partial fulfillment of the Degree of Master of Technology.

We further declare that this project work has not been submitted in full or part for the award of any other degree in this or any other Educational institutions

| B. Manikanta | 19MH5A0208 |
| --- | --- |

### ABSTRACT

Many tasks which are beyond human ability can be performed with the help of robotics. In this project, autonomous robots with a central navigation system are used to deliver packages from source to destination.

The navigation of tracking the robot using Arduino is used to deliver the objects from source to destination quickly within an effective manner with accuracy without using the sensors. The camera is continuously monitoring the robot and assigns the path based on the software instructions.

In this project, we have used a python script with an open cv to track the robot with color and used mathematics formulas to triangulate the color boxes on the robot to find the position and orientation of the robot. Thereafter we have used the python requests module to send the respective commands to navigate the robot from the pc. A single-camera has been used to track two robots. The pc will send the commands to Robot through wifi.

Autonomous robots can be used to improve the speed and accuracy of routine operations, particularly in warehousing and manufacturing spaces; work side-by-side with humans for added efficiency, and reduce the risk of employee injury in dangerous environments.

Robots are useful in many ways. For instance, it boosts the economy because businesses need to be efficient to keep up with the industry competition. Therefore, having robots helps business owners to be competitive, because robots can do jobs better and faster than humans can, e.g. robots can be built, and assemble a car.

Keywords : Arduino uno Wi-Fi rev 2, DC motors, webcam, Servo motor, Battery.

CONTENT

Table of Content                                                                         Page.No

1. Introduction                                                                                   1

  - Introduction                                                                             1

  - Electrical Aspects                                                                    2

  - Scope of Robots                                                                      3

  - Advantage                                                                               3

  - Limitations                                                                              4

1.6 Application                                                                              4

2. Literature Survey                                                                         5-6

3. Existing and Proposed Methods                                                    7

3.1 Existing Method                                                                      7

3.1.1 Line Follower Robot Circuit                                          7

3.1.2 Line Follower Robot Working                                       8

3.2 Proposed Method                                                                     9

3.2.1 Functional Block Diagram                                              9

4. Major Components                                                                        10

4.1 Arduino Uno Wi-Fi Rev2 Board                                             10

4.1.1 Details                                                                             11

4.1.2 Specifications                                                                  12

4.1.3 Details of Arduino Uno Wi-Fi Rev2 Board                  12-13

4.1.4 Use of Arduino Uno Wi-Fi Rev2                                    13

4.1.5 Applications of Arduino Uno Wi-Fi Shield                    13

4.2 Servo Motor                                                                               13

4.2.1 Servo Motor Theory                                                          14

### 4.2.2. Servo Motor Working Mechanism                                   15

4.2.3 Servo Motor Working Principle                                         15

### 4.2.4 Interfacing of Servo Motor With Microcontrollers            16

4.2.5 Controlling of Servo Motor                                             16-17

4.3 Webcam                                                                                       18

4.3.1 Technology                                                                          18

4.3.1.1 Image Sensor                                                           19

4.3.1.2 Optics                                                                       19

4.3.1.3 Compression                                                             19

4.3.1.4 Interface                                                                    20

4.3.1.5 Software                                                                    20

4.3.2 Characteristics                                                                      20-21

4.3.3 Uses                                                                                         21

4.3.3.1 Video monitoring                                                             21

### 4.3.3.2 Commerce                                                                  21

### 4.3.3.3 Video calling and video conferencing                     21-22

### 4.3.3.4 Video security                                                            22

### 4.3.3.5 Video clips and stills                                                  22

### 4.3.3.6 Input control devices                                               22-23

### 4.3.3.7 Astro photography                                                      23

### 4.3.3.8 Laser Beam Profile                                                    23

### 4.4 Gear Motor                                                                               23

### 4.4.1 Introduction                                                                  23-25

### 4.4.1.1 Applications                                                         26

### 4.5 Rechargeable Battery                                                               26

### 4.5.1 Rechargeable Battery                                                   26-27

4.5.2 Charging and Discharging                                               27

4.5.3 Rate of Discharge                                                             27

4.5.4 Damage from Cell Reversal                                        28-29

4.5.5 Damage during storage in fully discharged state            29

4.5.6 Depth of Discharge                                                          30

4.5.7 Lifespan and cycle stability                                             30

4.5.8 Recharging Time                                                          30-31

4.5.9 Active Component                                                          31

4.5.10 Types                                                                          31-33

4.6 Motor Driver Shield                                                               33-34

4.7 USB                                                                                          34

4.7.1 Introduction                                                                    34

4.7.2 USB Type – A Plug                                                      35-36

4.7.3 USB Standard Connectors                                            37-39

4.7.4 USB 2.0                                                                         39-40

4.8 Arduino IDE Compiler                                                           41-43

4.9 Python                                                                                        43

4.9.1 Development Environment                                            43-44

4.9.2 Advantage of Python                                                        44

5. Result Analysis                                                                                   45

5.1 Working                                                                                        45

5.2 GitHub Link for Code                                                                   46

5.3 Result                                                                                             46

6. Conclusion and Future Scope                                                              47

6.1 Conclusion                                                                                     47

6.2 Future Scope                                                                                  47

Reference

LIST OF FIGURES

### FIGURES                                                                              PAGE NUMBERS

3.1 . Line Follower Robot Circuit                                                                         7

3.2 . Working of IR Sensor                                                                                    8

3.3 . Block Diagram                                                                                               9

4.1 . Arduino Uno Wi-Fi Rev 2 Board                                                                11

4.2 . Servo Motor                                                                                               14

4.3 . Timing Diagram                                                                                         14

4.4 . Connections of Servo Motor                                                                          16

4.5 . Controlling of Servo Motor                                                                            17

4.6 . ASUS Webcam C3                                                                                         18

4.7 . Basic Gear Motor                                                                                            24

4.8 . Gear Motor                                                                                                      25

4.9 . Rechargeable Battery                                                                                      27

4.10 . Battery Recharge and Discharge Time                                                          28

4.11 . Ragone plot of common types                                                                       31

4.12 . Motor Driver Shield                                                                                       34

4.13 . High Speed USB Logo                                                                                   39

4.14 . A USB 2.0 PCI Expansion                                                                             39

4.15 . Arduino IDE Editor                                                                                        42

5.1 . Camera setup                                                                                                    45

5.2 . Robots                                                                                                               45

5.3 . Final Output                                                                                                      46